Q1: Multiple answer questions

| In WEP authentication, the terminal responds to a challenge with: | - The hash of the challenge and the secret key K - The encryption of the challenge with the secret key K - The HMAC of the challenge with the secret key K - The modular exponentiation of the key to the challenge |
| --- | --- |
| In AES-GCM, the hash function used for integrity is | - MD5 - SHA1 - SHA256 - Other: specify_____________________ |
| In cellular systems, the AUTN has the following role: | - Authenticate the base station - Authenticate the user - Guarantee location privacy - Signal the configuration of the network |
| A message authentication tag constructed as H(secret, message) is vulnerable to: | - Hash collisions - State precomputation - Message expansion - It is NOT vulnerable |
| If a block cipher uses a block size of 16 bits, what would be, in principle, the possible value(s) for the key? | - At most 16 bit - At most 16 bytes = 128 bit - At most 216 = 65536 bit - Can be more than 216 = 65536 bit |
| A server reply to a radius access-request message is authenticated as follows: | - An MD5 hash of the user request message - An MD5 hash of the server reply message - An HMAC MD5 of the user request message - An HMAC MD5 of the server reply message |
| Which type of nonce is used in TLS during data transfer? | - Time stamps - Sequence numbers - Random numbers - A combination of random and time stamp |
| The beast attack applies to which cipher? | - All block ciphers - Only to block ciphers in the AES family - Only to block ciphers using CBC - Only to block ciphers using ECB |
| The number of hash computations necessary to construct a full Merkle tree with 8 leafs is… | - 7 - 8 - 15 - 16 |
| HDKF constructs the i-th block as follows: | - HMAC(context, i) - HMAC(context, HMAC previous block i-1) - HMAC(…HMAC(HMAC(context, seed))) i times - HKDF does not use HMAC |
| We refer to Traffic Flow confidentiality as approaches which permit to: | - Protect a traffic flow from MITM attacks - Protect a traffic flow from packet replay attacks - Protect a traffic flow from statistical analysis - Protect a traffic flow via authenticated encryption |
| In IPsec ESP in tunnel mode… | - The inner IP header is integrity protected & encrypted - The inner IP header is only integrity protected - The outer IP header is integrity protected & encrypted - The outer IP header is only integrity protected |

Q2: A challenge response authentication system uses a block cipher in CBC mode. The authentication scheme is based on the CBC encryption of the challenge message. Assume block sizes of 8 bits and challenges also of 8 bits. An attacker sees the following legitimate exchange from a victim:

ß challenge = 1001.0110

Victim Response à (1000.1000) 0110.1110

The attacher now aims to impersonate the victim in a subsequent authentication. If the server sends a new challenge

ß newchallenge = 1001.1100

Which response should the attacker spoof to successfully authenticate?

(show either the spoofed message as well as the approach used to compute such message)

Q3: Using the extended Euclidean algorithm, compute the modular inverse 13-1 mod 41

Q4: Briefly illustrate the main difference between a Relay and a Redirect agent in Diameter

Q5: Describe the Ephemeral Diffie Hellman Key agreement and discuss why it provides forward secrecy.

Q6. (TLS Padding Oracle Attack) – Assume a CBC-based block encryption scheme which uses block sizes of 4 bytes each. Assume that the attacker sees the following ciphertext comprising 6 blocks (hex notation):

f1 aa bb cc   ||   10 01 02 20   ||   01 02 03 04  ||   12 23 34 45  ||  65 66 11 11  ||  ff f0 00 10

Assume now that the server is vulnerable to a Padding Oracle attack, and assume that the third block (the one underlined) contains a secret code which is either:

88 88 88 88            or              88 00 88 88

How many messages should the attacker send to the server/oracle to as to decrypt the code? And which specific modified ciphertext shall the attacker send?